# Azure Subnet Design for LZ

Core Principle

Subnets are security, routing, and operational boundaries, not just IP containers. If security, routing, scale, or lifecycle differs, create a new subnet.

Purpose-Driven Subnet Design

Design subnets by function: workloads, shared services, security appliances, and platform services. One subnet should serve one clear responsibility.

Address Space Planning

Use RFC1918 address space only (10/8 preferred). Allocate large VNETs, carve right-sized subnets, and leave 25–40% growth headroom.

Avoid IP overlap for hybrid connectivity.

Platform-Reserved Subnets

Certain services require dedicated subnets: Azure Firewall, VPN/ExpressRoute Gateway, Bastion, Application Gateway, and AKS node pools.

Never place workloads in these subnets.

Subnet Sizing

Azure reserves 5 IPs per subnet. Use /24 for application and shared services, /27–/26 for gateways, and larger ranges for AKS to support scaling.

Security Boundaries (NSGs)

Apply NSGs at the subnet level to define security zones. Use NIC-level NSGs only for exceptions. Always use explicit inbound and outbound rules.

Routing & Traffic Flow

Different routing requirements mean different subnets. Apply UDRs per subnet. Use default routes only when intentionally controlling egress.

Hybrid & DNS Awareness

Plan for on-prem connectivity, Private Endpoints, and DNS resolution paths early. Many hybrid issues stem from subnet design, not DNS configuration.

Isolation & Blast Radius

Use subnets to limit lateral movement and contain failures. Separate prod/non-prod and isolate sensitive or internet-facing workloads.

Naming & Automation

Use predictable, role-based subnet names and manage everything via IaC (Terraform). Design must be reproducible, policy-compliant, and automation-friendly.